- Emergency = 000 or 112

- 131 444 = nearest Police

- More numbers in the back of Bib Track books

- Sat phones will be on from 7-7:15 am & pm

TURNING THE PHONE ON

- Ensure the antenna is firmly in place.  If not there is a button
with an arrow (<) on top of the phone.  There is a line on the 
antenna.  Line up the < with the line (so the antenna points 
down along the back of the phone). Press the < button down 
and push the antenna in, then release the < button.

- The power button is in bottom left corner (Red circle with a dot). 
Press and hold until phone beeps on.

- Enter pin “1111” and press “OK” button 
(top right hand corner with Green writing).

- Rotate antenna until it clicks, then extend antenna.

- NB. When you are holding the phone whilst making a call 
the antenna should be pointing straight up.

IRIDIUM SAT PHONE TO LANDLINE

Dial: 00 61 8 1234 5678

00             for all outbound calls from the satellite phone

61             Australia country code

8               Area code (DO NOT dial leading zero from area code)

1234 5678 Actual phone number.

IRIDIUM SAT PHONE TO MOBILE

Dial:  00 61 412 345 678

00                for all outbound calls from the satellite phone

61                Australia country code

412 345 678 DO NOT dial leading zero

IRIDIUM SAT PHONE TO IRIDIUM SAT PHONE

Dial:  00 8816 555 55555

00                      for all outbound calls from the satellite phone

8816 555 55555 Satellite phone number

LANDLINE TO IRIDIUM SAT PHONE

From the Australian landline phone, dial  0011 8816 555 55555

0011                   International Direct Dial (IDD) code for Australia

8816 555 55555 the phone number for IRIDIUM phone.